| Metric |  | Blocking-Cache 1KB | Blocking-Cache + Store Buffer 1KB | Blocking-Cache + SB+RB 1KB |
| --- | --- | --- | --- | --- |
| Coremark | Cache hit rate | 69.04% | 85.71% | 87.3% |
|  | Cach miss rate | 30.96% | 14.29% | 12.7% |
|  | Store buffer hit rate | N/A | 5.5% | 5.5% |
|  | Replace Buffer hit rate | N/A | N/A | 8.79% |
|  | Number of cycle clock | 11126 | 10987 | 10848 |
|  | CPI | 1.6 | 1.58 | 1.56 |
|  | Number Instructions |
| 5 | Program execution time | 70427ns | 69547ns | 68667ns |
| Matrix Multiplication 32 X 32 | Cache hit rate | 77.56% | 79% | 79.6% |
|  | Cach miss rate | 22.44% | 21% | 20.4% |
|  | Store buffer hit rate | N/A | 8.31% | 5.27% |
|  | Replace Buffer hit rate | N/A | N/A | 3.12% |
|  | Number of cycle clock | 5259 | 4900 | 5130 |
|  | CPI | 1.61 | 1.5 | 1.57 |
|  | Number Instructions | 3267 |  |  |
|  | Program execution time | 33928ns | 33365ns | 32473ns |
| Quick Sort (đệ quy) | Cache hit rate | 60.8% | 79.1% | 83.46% |
|  | Cach miss rate | 39.2% | 20.9% | 16.54% |
|  | Store buffer hit rate | N/A | 7.69% | 7.69% |
|  | Replace Buffer hit rate | N/A | N/A | 4.8% |
|  | Number of cycle clock | 890 | 865 | 808 |
|  | CPI | 1.67 | 1.62 | 1.52 |
|  | Number Instructions | 531 |  |  |
|  | Program execution time | 12665ns | 12320ns | 12010 |
| Binary Search | Cache hit rate | 70.12% | 74.5% | 80.6% |
|  | Cach miss rate | 29.88% | 25.5% | 19.4% |
|  | Store buffer hit rate | N/A | 3.2% | 3.2% |
|  | Replace Buffer hit rate | N/A | N/A | 4.35% |
|  | Number of cycle clock | 440 | 410 | 395 |
|  | CPI | 1.647 | 1.53 | 1.47 |
|  | Number Instructions | 267 |  |  |
|  | Program execution time | 2785ns | 2595ns | 2500ns |

| Metric |  | Blocking-Cache + Store Buffer + RB 1KB | Blocking + SB + RB 4KB | Blocking-Cache + Store Buffer + RB 8KB | Blocking-Cache + Store Buffer + RB 32KB |
| --- | --- | --- | --- | --- | --- |
| Coremark | Cache hit rate | 87.3% | 89% | 91.8% | 95.4% |
|  | Cach miss rate | 12.7% | 11% | 8.2% | 4.6% |
|  | Store buffer hit rate | 5.5% | 3.5% | 2.5% | 2% |
|  | Replace Buffer hit rate | 8.79% | 3.7% | 2.67% | 2.4% |
|  | Number of cycle clock | 10848 | 10421 | 10315 | 9109 |
|  | CPI | 1.56 | 1.5 | 1.48 | 1.31 |
|  | Number Instructions | 6954 | 6954 | 6954 | 6954 |
| 5 | Program execution time | 68667ns | 65965ns | 65293ns | 57659ns |
| Matrix Multiplication 32 X 32 | Cache hit rate | 79.6% | 83.36% | 85.4% | 89% |
|  | Cach miss rate | 20.4% | 16.64% | 14.6% | 11% |
|  | Store buffer hit rate | 5.27% | 4.57% | 3.57% | 3.2% |
|  | Replace Buffer hit rate | 3.12% | 2.78% | 2.6% | 2.5% |
|  | Number of cycle clock | 5130 | 4769 | 4573 | 4443 |
|  | CPI | 1.57 | 1.46 | 1.4 | 1.36 |
|  | Number Instructions | 3267 | 3267 | 3267 | 3267 |
|  | Program execution time | 32472ns | 30187ns | 28952ns | 26192ns |
| Quick Sort (đệ quy) | Cache hit rate | 83.46% | 86.6% | 88% | 92.5% |
|  | Cach miss rate | 16.54% | 13.4% | 12% | 7.5% |
|  | Store buffer hit rate | 7.69% | 5.23% | 5.2% | 3.5% |
|  | Replace Buffer hit rate | 4.8% | 4.3% | 3.1% | 2.6% |
|  | Number of cycle clock | 808 | 759 | 717 | 695 |
|  | CPI | 1.52 | 1.43 | 1.35 | 1.31 |
|  | Number Instructions | 531 | 531 | 531 | 531 |
|  | Program execution time | 12010ns | 11675ns | 11346ns | 9839ns |
| Binary Search | Cache hit rate | 80.6% | 85.4% | 87.15% | 93.8% |
|  | Cach miss rate | 19.4% | 14.6% | 12.85% | 6.2% |
|  | Store buffer hit rate | 3.2% | 2.55% | 2.4% | 2.3% |
|  | Replace Buffer hit rate | 4.35% | 3.24% | 3% | 2.12% |
|  | Number of cycle clock | 395 | 373 | 354 | 342 |
|  | CPI | 1.47 | 1.4 | 1.32 | 1.28 |
|  | Number Instructions | 267 | 267 | 267 | 267 |
|  | Program execution time | 2500ns | 2361ns | 2240ns | 2144ns |